Method and system for controlling the frequency of the supply voltage for ultrasonic piezoelectric power transducers

Patent Number(s): RO128368-A2

Inventor(s): VOLOSENCU C

Patent Assignee Name(s) and Code(s): VOLOSENCU C(VOLO-Individual)

Derwent Primary Accession Number: 2013-K35905 [44]

Abstract: NOVELTY - The invention relates to a method and a system for controlling the frequency of the supply voltage for the ultrasonic piezoelectric power transducers. According to the invention, the method is based on estimating the motion current derivative and using the estimation in a control loop depending on the error, so that there is provided the maximum mechanical power developed by the transducer, based on the measurement of the transducer voltage and current, by controlling the frequency of the supply voltage, in order to maintain the motion current derivative to zero. As claimed by the invention, the system comprises a power amplifier (AP) operating in commutation at high frequency, which supplies a piezoelectric transducer (E) with periodical electric voltage directly or by means of a voltage step-up (T), the voltage and the current of the transducer (E) are measured by means of a voltage transducer (Tu), and a current transducer (Ti), respectively; an estimating current (CC_DCM) uses the measured voltage and current values for estimating the motion current derivative, the output thereof is connected to the inverting input of a frequency control circuit (RG-f) of PI type which operates depending on the error between the motion current estimation and zero, the output of the frequency controller (RG-f) is connected to the voltage control input of a voltage-controlled oscillating current (GF_CT) which generates an impulse sequence with a frequency proportional to the control voltage provided by the controller (RG-f), and the output of the voltage-controlled frequency generator (GF_CT) is connected to a control circuit (CC) of the power amplifier (AP).

International Patent Classification: B06B-001/02; H01L-041/02; H03J-007/02

Derwent Class Code(s): P43 (Sorting, cleaning, waste disposal); V06 (Electromechanical Transducers and Small Machines)

Derwent Manual Code(s): V06-V01B; V06-V01N; V06-V02S; V06-V04C

Patent Details: Patent Number; Publ. Date; Main IPC; Week; Page Count; Language. RO128368-A2; 30 May 2013; B06B-001/02; 201344; Romanian.

Application Details: RO128368-A2; RO001024; 14 Oct 2011.

Priority Application Information and Date: RO001024; 14 Oct 2011.

Top of Form

Method of correction for fuzzy logic controllers is to provide control systems stability. Patent Number(s): RO128437-A2. Inventor(s): VOLOSENCU C. Patent Assignee Name(s) and Code(s): VOLOSENCU C(VOLO-Individual). Derwent Primary Accession Number: 2013-K35684 [38]. Abstract: NOVELTY - The invention relates to a correction method for fuzzy logic-based controllers to provide control systems stability. According to the invention, the method is based on the elimination of a fuzzy block non-intervention zone, positioning the fuzzy block transfer characteristic in the quadrants I and III, according to the stability analysis circle method; this way the stability of control systems based on the fuzzy PI regulator is provided by correcting the quantity at the defuzzied output of the fuzzy block by a correction quantity obtained as the difference between the sum of limited inputs of the fuzzy block and the sum of unlimited inputs.. International Patent Classification: G05B-013/02; G06F-017/10. Derwent Class Code(s): T01 (Digital Computers); T06 (Process and Machine Control). Derwent Manual Code(s): T01-J04; T06-A05. Patent Details: Patent Number; Publ. Date; Main IPC; Week; Page Count; Language. RO128437-A2; 30 May 2013; G05B-013/02; 201338; Romanian.. Application Details: RO128437-A2; RO001023; 14 Oct 2011.. Priority Application Information and Date: RO001023; 14 Oct 2011..

This record is from: Derwent Innovations IndexSM